- EVK Overview

HPL-EVK V4R2 (EVK) is an evaluation kit designed for testing of high-precision boards and modules, and can be used with UB4B0, UB4B0M, UB482, UM4B0-EXTA, UM482-EXTA etc.

1

2

3

4

5

6

7

8

9

12

10

13

11

14

15

Note: Please make sure the board is installed and all the interfaces are connected normally before powering on the EVK. DO NOT plug/unplug the product when the power is on.

10

Fi

Figure 1. HPL-EVK Front

| - Power Connector | - Power Switch | - COM1 | - CAN |
| --- | --- | --- | --- |
| - COM3 | - Reset Button | - COM2 | - Ethernet Port |
| - USB-B Port | - SMA Ports (total 6) | - Dial Switch | - SPI & UART ½ |
| - SPDT Switch | - NII_UART0/1 (LVTTL) | - NII_UART0/1 (RS232) |  |

- Instructions

In the same order as in Figure 1.

- Power Connector – Recommended power range is 9-36V. The power supply above 36V may cause permanent damage to EVK and/or the board.

- Power Switch – position to the left is OFF.

- COM1 – use the RS232 connector to link with computer.

- CAN – above the printed letters JP2
JP2 PIN definitions

| PIN1 | CANL |
| --- | --- |
| PIN2 | GND |
| PIN3 | CANH |
| PIN4 | GND |

- COM3 – use the RS232 connector to link with computer.

- Reset Button – press to reset the HPL-EVK.

- COM2 – use the RS232 connector to link with computer.

- Ethernet Port.

- USB-B Port.

- SMA Ports 
SMA Port PIN definitions (based on the letters nearby_

| J5 – EVT1 | J8 – PPS | J21 – FWD | J11 – SPEED | J6 – EVT2 | J20 – PPS2 |
| --- | --- | --- | --- | --- | --- |

- Dial Switch
PIN1 and PIN2 are used for boards functional selection.

| PIN2 | PIN1 | Functions |
| --- | --- | --- |
| 0 | 0 | EVENT2&GPIO |
| 0 | 1 | COM3 |
| 1 | 0 | CAN |
| 1 | 1 | Reserved |

Dial Switch PIN3 for whether the RTK signal is being received.

| PIN3 | Functions |
| --- | --- |
| 0 | RTK_LED signal is not received |
| 1 | RTK_LED signal is received |

- Reserved for SPI & UART ½ connector.

- SPDT switch – returns to factory settings. Normally should be kept in the right position on OFF.

- NII_UART0/1 (LVTTL) – reserved for LVTTL.

- NII_UART0/1 (RS232) – reserved for RS232.